File -> open -> choose the right .urdf file

Joint order:

- 0 shoulder_pan

- 1 shoulder_lift

- 2 elbow_flex

- 3 wrist_flex

- 4 wrist_roll

- 5 gripper

Add a robot to Stage

- Start with a new stage, File > New Stage.

- Add robot to the scene, from the top Menu Bar, click Create > Robots > Franka Emika Panda Arm.

Examine the robot

Use the Physics Inspector to examine the robot’s joint properties.

- Go to Tools > Physics > Physics Inspector. A window opens on the right.

- Select Franka to inspect. The window will populate the joint information, such as the upper and lower limits as well as its default position by default.

- Click on the hamburger icon on the top right to see more options, such as the joint stiffness and damping.

- Optionally, make any changes to these values to see the robot move on the Stage corresponding to the change. A green check mark will appear.

- To commit the changes to be the new default values for the robot, click the green check mark.

Control the Robot

The GUI-based robot controllers are inside the Omniverse visual programming tool, OmniGraphs. There are more involved tutorials about OmniGraph in the Omnigraph section. For the purpose of this tutorial, we will generate the graph using a shortcut tool, and then examine the graph in the OmniGraph editor.

- Open the graph generator by going to Tools > Robotics > Omnigraph Controllers > Joint Position.

- In the newly appeared Articulation Position Controller Inputs popup window, click Add for the Robot Prim field.

- Select Franka as the Target.

- Click OK to generate the graph.

To move the robot:

- In the Stage tab to the upper right, select Graph > Position_Controller.

- Select the JointCommandArray node. You can do this by either selecting the node on the Stage tree, or selecting the node in the graph editor.

- In the Property tab to the lower right, you can see the joint command values. The Inputs under the Construct Array Node correspond to joints on the robot, starting with the base joint.

- Click+hold+drag various value fields or type different values to see the robot arm change position.

- Press Play to start the simulation.